--------------------------------------------------------------------------------

--

--   AMD2910 Benchmark

--

-- Source:  AMD data book

--

-- author: Indraneel Ghosh

--         University Of California, Irvine, CA 92717

--

-- Written on Feb19, 1992

--

--------------------------------------------------------------------------------

TESTING THE Am2910 MODELS.

PORTS

I : in

CCEN_BAR : in

CC_BAR : in

RLD_BAR : in

CI : in

OEbar : in

clk : in

D : in

Y : out

PL_BAR : out

VECT_BAR : out

MAP_BAR : out

FULL_BAR : out

Details about these ports are mentioned in the "am2910.doc" file.

OVERALL TESTING STRATEGY

In testing the Am2910 models, the overall strategy adopted is to

test each "hardware" component (e.g. stack, register/counter etc.)  using

sequences of test vectors.

The components are tested in different modes with test vectors designed

to detect "stuck-at-0" and "stuck-at-1" faults at various points in the

hardware.

Because the components are not being tested in isolation, we need to

set up input values at input ports of the chip and propagate them to

the input of that component. Also, the output of a component has to

be propagated to the output ports of the chip.

So, an error indication generated while running the test vectors for a

particular component might result from faults in other components that

are in the propagation paths.

However, the overall error indication pattern, generated by the simulation,

will provide a better pointer to the location of the fault. It will exclude

some possible fault locations, in most cases.

TESTING THE COMPONENTS

Now, we discuss the testing of the individual components.

**************************

*	                 *

* TEST VECTORS FOR STACK *

*       	         *

**************************

In testing the stack ( which has five levels of nesting), we push five

different words into the stack and monitor the word at the top of the

stack and the FULL_BAR signal in each case.

Then we do five pops and again monitor the word at the top of the stack

and the FULL_BAR signal at each stage.

To test for stuck-at faults in each stack location, we then repeat the

above test with five words that are Boolean complements of the ones

used in the above test.

We also, test for each set of conditions ( instruction, condition code

and register/counter value) that causes a PUSH or a POP.

*************************************

*	                            *

* TEST VECTORS FOR REGISTER COUNTER *

*       	         	    *

*************************************

Now, we test for stuck-at faults in the register/counter and its control

logic.

******* TEST LOADING OPERATION (INSTRUCTION) *******

To test loading operations, we load all 1's and all 0's, for each set of

conditions ( instruction, condition code and register/counter value) that

causes a load operation.

******* TEST LOADING OPERATION (RLD_BAR) *******

To test loading operations when signal RLD_BAR is asserted, we load all 1's

and all 0's into the register/counter.

******* TEST DECREMENT OPERATION ********

To test decrement operations, we perform one decrement operation, for each

set of conditions ( instruction, condition code and register/counter value)

that causes a decrement operation.

******* TEST DECREMENT OPERATION BORROW RIPPLE *******

We run test vectors to test the rippling of borrow through each of the

12 bits in the decrement operation.

*****************************************

*	                                *

* TEST VECTORS FOR MICROPROGRAM COUNTER *

*       	         	        *

*****************************************

Now, we test for stuck-at faults in the microprogram counter and its control

logic.

******* TEST CLEARING OPERATION ********

We test the clearing operation of the uPC under all the relevant conditions.

******* TEST LOADING OPERATION ********

We load all 0's and all 1's to test for stuck at faults.

******* TEST INCREMENT CARRY RIPPLE ********

We run test vectors to test the rippling of carry through each of the

12 bits in the increment operation.

********************************

*	                       *

* TEST VECTORS FOR MULTIPLEXER *

*              	               *

********************************

We run test vectors to test for stuck-at-0 and stuck-at-1 faults under

all the relevant set of conditions (instruction, condition code,

register/counter value) in the following cases :

MULTIPLEXER SELECTS 0

MULTIPLEXER SELECTS D

MULTIPLEXER SELECTS R

MULTIPLEXER SELECTS uPC

MULTIPLEXER SELECTS TOS

*************************************

*	                            *

* TEST VECTORS FOR TRI-STATE-DRIVER *

*              	                    *

*************************************

We test the tri-stating of outputs under the control of signal OEbar.

*************************************

*	                            *

* TEST VECTORS FOR ENABLE SIGNALS   *

*              	                    *

*************************************

We test the assertion of signals PL_BAR, VECT_BAR and MAP_BAR under

different sets of conditions ( instruction, condition code and

register/counter contents).