--------------------------------------------------------------------------------

--

--   Intel 8251 Benchmark

--

-- Source:  Intel Data Book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on April 7, 92

--

--------------------------------------------------------------------------------

TESTING THE 8251 MODEL

OVERALL TESTING STRATEGY

In testing the functionality of the "main" process, we mainly

concentrate on testing its main operational modes :

(1) Write various mode words into the "main" process and monitor the

values of global signals computed by it.

(2) Write SYNC characters (both Single-Sync and Double-Sync) into the

"main" process and test whether they are correctly stored.

(3) Write different Command words into the "main" process and monitor

the execution of commands like internal reset, error reset etc.

(4) Write data for transmission into the "main" process

(5) Read received data from the "main" process

(6) Read status from the "main" process

--------------------------------------------------------------------------

FORMAT OF TEST-VECTORS IN "*.bits" FILE ( "bit_format" )

In this file, the test vectors for the "main" process of the 8251

USART are stored in a format known as the"bit_format". These test vectors

can be very easily converted to other HDL formats. I have a C program that

converts them into VHDL format.

The "bit_format" has the following characteristics :

1) This file is line-oriented and there are three types of lines :-

[a] Port Description Lines

[b] Actual Test Vector Lines

[c] Comment Lines.

2) All comment lines (Type [c]) begin with a star (*). While translating

these test vectors into some HDL format (e.g. VHDL), the initial comment

lines are ignored. All other comment lines are copied verbatim into the

HDL file.

3) In lines of Types [a] and [b], the line has many fields corresponding to

ports of the entity being tested. A colon (:) acts as a field-separator

between ports ( We will call it a port separator). Each such line must

end with a colon.

4) There are three Port Description Lines ( Type [a] ). These lines come

just after the initial comment lines ( of Type [c] ). The three lines

(in the following order) are :-

{I} Port Name Line : This line specifies the names of all ports of the

entity being tested. Port names are separated by a

colon.

The order in which the port names are specified is

significant. In all other lines of Type [a] and [b],

information pertaining to specific ports must appear

in the same order with a colon as a port-separator.

All input ports are mentioned first, followed by

the output ports. Inout ports are mentioned both

as input as well as output ports.

{II} Port Direction Line : This line specifies the directions ( in, out,

inout ) of the ports mentioned in the Port

Name Line ( Line {I} ).

{III} Port Type Line : This line is related to the data type ( e.g.

bit, bit-vector, integer etc.) of the ports

mentioned in the Port Name Line ( Line {I} ).

This line is very specific to VHDL data type

declaration syntax and may not be relevant to

other HDL's.

5) The Actual Test Vector Lines (Type [b] ) contain the test vectors for the

8251.

Each test vector line is equivalent to one clock cycle.

Values corresponding to ports are written in the exact order in which

port names are mentioned in the Port Name Line ( Type [a], Line {I} ).

In case we dont want to mention any value for a particular port in that

line, we put in one or more dashes ( - ) in that position corresponding

to that port. Values are separated by the colon as a port-separator.

Values in the input-port positions in Type [b] lines are assigned to the

corresponding input port. Only the "clocking" is implicit in the

translation process for each test vector and the clock ( "clk")

is not specifically mentioned among the input ports.

Values in the output-port positions in Type [b] lines are compared to the

actual output values appearing at the corresponding output ports when we

run the simulation. If these two values dont match, then an "Assert"

statement is generated to show that an error has been detected.

--------------------------------------------------------------------------

HOW TO ADD MORE TEST VECTORS

More test vectors can be easily added simply by entering more lines in

"bit_format" to the end of the file. These lines must be "Actual Test

Vector Lines ( Type [b] )".

"Comment Lines ( Type [c] )" may also be added, but those will not have

any effect on simulation.

--------------------------------------------------------------------------

TRANSLATION FROM "bit_format" TO VHDL.

In this directory, I have a C program called "trans_main.c" that does this

translation.

The following steps can be followed :-

(i) Compile the translator file.

"cc trans_main.c -o trans_main"

(ii) Run the translation program by specifying an input and an output file.

"trans_main sytx.bits sytx.vhdl"

This program accepts test vectors in "bit_format" from an input file

(for example) "sytx.bits" and produces test vectors in VHDL format in

output file "sytx.vhdl"